Instructions to adjust the TxF

The TX filter board is shown as following figure (line Hybrid is included)

TX Signal

(from TX Amplifier)

Hybrid

RX Signal

filter

RF Line Signal

Pin9

Pin11

Pin1

Pin3

Pin5

Pin6

filter

=

Input inductor1

Output inductor2

Capacity1

set by XP1

Capacity2 set by XP4

Capacity3

set by XP2 & XP3

| Pin1, 2 | Line Signal. The load resister is 75ohm |
| --- | --- |
| Pin3, 4 | Line Signal GND |
| Pin5 | RX Signal (+). The output resister is 150ohm |
| Pin6 | RX Signal (-). |
| Pin9, 10 | TX Signal. The input resister is 25ohm (16KHz filter) 16.5ohm(8KHz filter) 50ohm(48Khz filter) |
| Pin11, 12 | TX Signal GND |

The pin definition of TX filter board: (from top to bottom)

| Pin1, 2 | Line Signal. The load resister is 75ohm |
| --- | --- |
| Pin3, 4 | Line Signal GND |
| Pin5 | RX Signal (+). The output resister is 150ohm |
| Pin6 | RX Signal (-). |
| Pin9, 10 | TX Signal. The input resister is 25ohm. |
| Pin11, 12 | TX Signal GND |

The steps of adjust TX filter

In order to set TX filter conveniently, a DIN connector is needed.

The DIN type code is 09062486835

Set TX filter with a new central frequency as the following steps

Step1. Set the jumpers by means of the table of Central Frequency –Jumper positions. After this step, the filter amplitude fluctuation in band is less than 5dB. And the filter could work on setting band for UPLC-100. If the S/N is less than 35dB in lab, make more steps as following.

Step2. There are two inductors on TX filter. Adjust the input inductor (the one close to DIN connector) firstly.

Keep Line Signal (Pin1) and RX Signal pins open and connect a signal oscillator(50ohm) and level meter(high Z) as the figure. Set the Signal Oscillator/Generator as the required central frequency, and level is set to 0~20dBm. Adjust the input inductor and let the level meter to show the smallest value.

Pin9

Pin11

Hybrid

open

filter

Level meter

Signal Oscillator/

open

Pin1

Pin3

Pin5

Pin6

Step3: Then adjust the output inductor (the one close to panel) as the following figure.

Keep TX Signal(pin9) and RX Signal pins open. Set the output level Oscillator(50ohm) as 0~20dBm. Adjust the output inductor and let the level meter to show the smallest value.

Pin1

Pin3

Hybrid

open

filter

Signal Oscillator

Pin9

Pin11

open

Level meter

It is finished. TX filter could work well for digital PLC now (S/N>40dB ).

In order to get excellent fine setting for the filter, the following additional steps maybe required.

Connect a 16.5ohm resistor in the input of filter (Pin9 and Pin11).

Pin1

Pin3

Hybrid

open

filter

Pin9

Pin11

Return loss testing meter

16.5ohm

- Test the return loss at output of the filter. Shift or increase/decrease the two peaks. Make the two peaks balance and the central frequency in the central of return loss figure. Adjust the Q value of the filter and make the width of return loss figure just reach the requirement.

- Check the tapping loss figure.

- Insert the filter into chassis and repeat the step 4-5 until reach the requirement.

The input/output inductors and the capacity set by XP2 & XP3 are required to be adjusted for fine setting.

| The object of adjustment | Operation |  |  |
| --- | --- | --- | --- |
|  | Input Inductor (close to the connector) | Output Inductor (close to the front panel) | The capacitor controlled by XP2-XP3 |
| Shift the return loss figure to lower frequency direction | Anti-clockwise direction | Anti-clockwise direction |  |
| Shift the return loss figure to higher frequency direction | Clockwise direction | Clockwise direction |  |
| Raise the left peak and lower the right peak. (Tip 1) | Clockwise direction | Anti-clockwise direction |  |
| Lower the left peak and raise the right peak | Anti-clockwise direction | Clockwise direction |  |
| Increase Q value of filter (Tip2 & Tip3) |  |  | Increasing the capacitor. |
| Decreasing Q value of filter |  |  | Decreasing the capacitor |

Tip1: It is the best that there are two balanced peaks on return loss figure.

Tip2: The increase Q of filter will result in: (1) The width of return loss and tapping loss figures are increased. (2) The depth of the dip between the dual peaks becomes shallower.

Tip3: It is better to increasing width of the return loss figure and decreasing width of tapping loss figure. However when width of return loss figure is increased, the tapping loss figure is increased too. So we must select suitable width of return loss figure.